|  | Appendix A USB & the SDT170 |
| --- | --- |

DO NOT CONNECT THE USB –SERIAL CONVERTER

CABLE BEFORE INSTALLING THE SOFTWARE DRIVER!

System requirements.

- Windows 98, Windows ME, Windows 2000, or Windows XP operating system.

- AMD or Intel Pentium 133MHz or better based computer.

- USB 1.1 or higher port.

Installing the USB driver software.

Insert the CD-ROM into the drive.

Open the CD-ROM via explorer or via “This computer”.

Double click USB DRIVER, the installation will

start.

Click “Next” to proceed.

To finalise the installation click “Finish”.

If the USB driver is installed the SUB-serial converter cable can be connected. Windows XP or Windows 2000 will recognise the cable immediately. The following messages will appear:

For your own convenience always connect the cable to the same USB port of your PC. Windows generates automatically a com port for the connected cable. So, if you switch to another USB port, the com port number will be different and you have checked the number in the Windows device manager and change every time the “Com port setup” in the SDT software. By using the same USB port, the installation is made once and for all.

Checking the com port.

Right mouse clicks on “This computer”.

Following menu appears.

Select “Manage”.

Select “Device Manager”.

Scroll in the right section of the window to “Port (Com & LPT)”.

Click on the litlle corss and “Profilic USB-to-Serial Comm Port (COM5)” appears.

Windows generated COM 5 for the cable.

This number was selected arbitrarily by your PC.

Checking the com port settings.

Double click “Profilic USB-to-Serial Comm Port”.

The window with the com port properties is displayed.

Go to “Port Settings”.

Port Settings:

Bits per second: 9600

Data bits: 8

Parity: None

Stop bits: 1

Flow Control: None.

If one of these setting is not correct, change with the little drop down menu.

If all the settings are correct click “OK”.

Changing the com port number.

Click by “Port settings” on “Advanced”.

In the window diplayed, you

will find the number of the com port.

If this one corresponds to that of

your application, click on “OK”.

If not, change for the number of

the com port of your application before clicking on “OK”.