|  | Appendix A USB & the SDT170 |
| --- | --- |

DO NOT CONNECT THE USB –SERIAL CONVERTER

CABLE BEFORE INSTALLING THE SOFTWARE DRIVER!

System requirements.

- Windows Vista operating system.

- AMD or Intel Pentium 133MHz or better based computer.

- USB 1.1 or higher port.

Installing the USB driver software.

- Extract the zip file EM1016-INF.zip

- Un-plug the usb cable from your computer

- Un-install your actual RS232/USB adapter driver from windows control panel (and wait until it completely finished)

- Plug the usb cable into your computer: Windows will detect a new device, please select to install the driver manually; and select the folder you have obtained at first step.

(Vista will maybe ask you to confirm that you want to install an “un-signed” driver, please confirm with “yes”).

Checking the com port.

Right mouse clicks on “This computer”.

Following menu appears.

Select “Manage”.

Select “Device Manager”.

Scroll in the right section of the window to “Port (Com & LPT)”.

Click on the litlle corss and “Profilic USB-to-Serial Comm Port (COM5)” appears.

Windows generated COM 5 for the cable.

This number was selected arbitrarily by your PC.

Checking the com port settings.

Double click “Profilic USB-to-Serial Comm Port”.

The window with the com port properties is displayed.

Go to “Port Settings”.

Port Settings:

Bits per second: 9600

Data bits: 8

Parity: None

Stop bits: 1

Flow Control: None.

If one of these setting is not correct, change with the little drop down menu.

If all the settings are correct click “OK”.

Changing the com port number.

Click by “Port settings” on “Advanced”.

In the window diplayed, you

will find the number of the com port.

If this one corresponds to that of

your application, click on “OK”.

If not, change for the number of

the com port of your application before clicking on “OK”.